# Setup steps for SSH connections to AWS CodeCommit repositories on Window

STEP 1:

- Open Command Prompt

- Type the command :ssh-keygen and follow the directions to save the file to the .ssh directory for your profile.

The Public and Private Key will generate in the .ssh Path

STEP 2:

Go to ssh key Path and Copy the Public key

- And Select IAM USER from the IAM Console.Select the Security Credentials option on the user details page, and then select Upload SSH public key..

- Paste the contents of your SSH public key into the field, and then choose Upload SSH public key.

STEP 3:

- After Paste it status will active and copy the SSH Key ID

- And SSH Key ID Paste in Config file and  identity file path give public and private key name.

STEP 4 ;

- Run the command to test your SSH configuration :ssh git-codecommit.us-east-2.amazonaws.com (It is ssh key link of codecommit)

- To troubleshoot problems, run the ssh command with the -v parameter

STEP 5 :

- Download the tortoise git Download TortoiseGit 2.14.0.1 64-bit (21.6 MiB)

- In Installing Process Select the Open SSH

- Connect to the CodeCommit console and clone the repository and Copy the SSH clone Paste in TortoiseGit